ACN Chapter 1 – Network Layer & Protocols

*MSBTE, Computer Engineering Semester 5*

1. Network Layer: Role and Responsibilities

What is the Network Layer?

- Responsible for host-to-host delivery of packets across diverse networks.

- Layer 3 in the TCP/IP 5-layer model.

- Interfaces: Receives from Data Link Layer, provides to Transport Layer.

Key Duties

- Internetworking: Logical connection of various networks.

- Addressing: Assigning unique IP addresses to interfaces/devices.

- Routing: Selecting optimal path for data.

- Packetizing:  Encapping transport layer data into packets.

- Fragmentation:  Dividing packets based on network requirements.

2. IPv4 Addresses

Characteristics

- 32-bit  identifier, assigned to every network interface.

- Notations: Binary, Dotted Decimal, Hexadecimal.

Notation Types

Address Space

- Total: 2^{32} = 4,294,967,296 unique addresses.

Example (Dotted Decimal Conversion)

- Binary: `10010001 00001010 00100010 00000011`

- Decimal: `145.10.34.3`

3. Classful Addressing

IPv4 Classes

Identifying Classes:

A: 0 – 127

B: 128 – 191

C: 192 – 223

D: 224 – 239

E: 240 – 255

Two-Level Addressing

Network Address Extraction:

-  Net ID:  Identifies network

-  Host ID:  Host within network

- Example:

- Address: 132.7.21.84 (Class B)

- Net ID: 132.7

- Host ID: 21.84

Default Network Masks

| Class | Default Mask      |

|------- |-------------------|

| A       | 255.0.0.0         |

| B       | 255.255.0.0       |

| C       | 255.255.255.0     |

Extracting Network Address:

Apply bitwise AND between IP and mask.

Example: 132.7.21.84 AND 255.255.0.0 = 132.7.0.0

4. Subnetting (Three-Level Addressing)

- Subnetting  divides large networks into smaller subnets.

- Requires creating a  subnet mask  with added bits.

Example (Class C):

- Original: 192.168.1.0/24 (mask: 255.255.255.0)

- 8 subnets: Need 3 extra ones (mask: 255.255.255.224, or /27)

5. Special IP Addresses

6. Limitations of IPv4

-  Address Exhaustion:  32-bit pool is limited.

-  Management Difficulties:  Rigid hierarchical structure.

-  No mobility or built-in security.

7. Classless Addressing (CIDR)

- Abandons fixed classes—allocates variable-size blocks using “slash notation” (e.g., 192.168.1.0/26).

- More efficient address use; supports blocks for small/large needs.

| CIDR Notation     | Block Size        | Network Mask        |

|------------------|-------------------|---------------------|

| 192.168.1.0/24   | 256 addresses     | 255.255.255.0       |

| 192.168.1.0/26   | 64 addresses      | 255.255.255.192     |

| 192.168.1.0/27   | 32 addresses      | 255.255.255.224     |

Example:

- Block: 64.32.16.8/27

- Block size: $$2^{32-27} = 32$$ addresses

- Range: 64.32.16.0 to 64.32.16.31

- Mask: 255.255.255.224

8. Supernetting

- Combines multiple (usually Class C) blocks into one "supernet" for larger organisations.

9. Registered vs. Private (Unregistered) IPs

| Type         | Usage                   | Registration       |

|--------------|------------------------|--------------------|

| Registered   | Public Internet         | Yes (via IANA/ISP) |

| Unregistered | Private LANs/Offices    | No (local only)    |

10. NAT (Network Address Translation)

- Solves limited public IP space by mapping internal private IPs to one public IP externally.

- Internal addresses are not routable on the public Internet.

- Used widely in company LANs.

11. IPv4 Datagram & Header Format

12. Fragmentation & MTU

- Different networks have different  MTUs  (Maximum Transmission Units).

- IP datagrams may be split (fragmented) to fit these MTUs.

- Each fragment travels independently and is reassembled at destination.

13. IPv4 Header Options

-  Single-Byte:  No Operation, End of Options

-  Multi-Byte:  Record Route, Source Routing (Strict/Loose), Timestamp

14. ICMPv4 – Error Control & Diagnosis

- Used for error messages and query functions.

-  Error:  Destination Unreachable, Time Exceeded, Source Quench, etc.

-  Query:  Echo (Ping), Timestamp

ICMP Message Format:

- Includes type, code, checksum in header.

15. Mobile IP

-  Enables portable devices  to maintain the same IP address across networks.

- Uses two IPs:  Home Address  (permanent),  Care-of Address  (temporary).

-  Agents:  Home Agent (HA), Foreign Agent (FA)

-  Phases:  Discovery, Registration, Data Transfer

16. Virtual Private Network (VPN)

- Extends private networks securely over public infrastructure (like the Internet).

- Uses encryption and tunneling protocols to protect data.

17. Key Network Layer Protocols (Summary Table)

| Protocol | Purpose                    |

|----------|----------------------------|

| IP          | Packet delivery, addressing|

| ICMP    | Error reports, diagnostics |

| NAT      | Maps private↔public IPs    |

| VPN      | Secure remote connectivity |

18. Additional Examples

-  Finding class of address:

If first octet = 24 → Class A

If first octet = 221 → Class C

-  Subnetting Example:

- Network: 192.168.1.0/24

- 4 subnets: Need 2 more bits → /26 mask (255.255.255.192)

- Subnet address ranges:

- 192.168.1.0–63, 64–127, 128–191, 192–255