Extracting Information in a Block (Classful Addressing)

Introduction

In IPv4 classful addressing, an address block is a contiguous range of addresses assigned for use within a network or subnet. Extracting key details from a block helps network administrators determine usability, subnetting, and efficient address allocation.

Steps to Extract Information from a Block

1. Identify the Class of the Address

- Check the first octet of the IPv4 address.

- Class A: First octet 1–126

- Class B: First octet 128–191

- Class C: First octet 192–223

2. Find the Net ID Length (n)

- The net ID bits (n) differ by class:

- Class A: n = 8 bits

- Class B: n = 16 bits

- Class C: n = 24 bits

3. Total Number of Addresses in the Block

Calculate using:

N = 2^{(32-n)}

Where  n  is the number of network prefix bits.

- Class A:** 2^{24} = 16,777,216 addresses

- Class B:** 2^{16} = 65,536 addresses

- Class C:** 2^{8} = 256 addresses

4. First Address in the Block (Network Address)

- Set all Host ID bits to 0; keep Net ID the same as the original address.

- *Example:*

- For class B address `151.15.67.63`, replace the host portion (`67.63`) with `0.0`:

- **Network Address:** `151.15.0.0`

5. Last Address in the Block

- Keep the Net ID as is; set all Host ID bits to 1.

- *Formula:*

- Last address = Network address + (total number of addresses - 1)

- *Example:*

- **First Address:** `151.15.0.0`

- **Last Address:** `151.15.255.255` (for class B, since host part is 16 bits)

6. Calculating Usable Host Addresses

- Usable addresses exclude the network (all host bits 0) and broadcast (all host bits 1) addresses. For most networks:

- Usable = Total − 2

Example

Given:** Address 24.46.8.95

- Class A (first octet 24)

- Net ID = 8 bits, Host ID = 24 bits

- First Address: 24.0.0.0

- Last Address: 24.255.255.255

- Total addresses: $$2^{24} = 16,777,216$$