CHAPTER 1 – SOLVED EXAMPLES (STEP-BY-STEP)

Ex. 1.3.1 – Identify Network Type & Network Address

Question: For the address `24.46.8.95`, identify the type of network and find the network address.

Steps:

1. First byte = `24`.

2. Compare with IPv4 class ranges:

- Class A: 0–127

- Class B: 128–191

- Class C: 192–223

3. `24` → Falls in Class A.

4. In Class A:

- Net ID = first byte

- Host ID = remaining 3 bytes

5. Network address = Keep Net ID as is, replace Host ID with 0’s → `24.0.0.0`.

Final Answer: Class A**, Network address: `24.0.0.0`

Ex. 1.3.2 – Identify Type & Network Address**

Question: For the address `132.7.21.84` find the type of network and network address.

Steps:

1. First byte = `132`

2. Range 128–191 → **Class B**

3. Net ID = First two bytes → `132.7`

4. Replace Host ID with zeros → `132.7.0.0`

Final Answer: Class B**, Network address: `132.7.0.0`

Ex. 1.3.3 – Identify Class

Question: Find the class of the address `221.46.75.64`.

Steps:

1. First byte = `221`

2. Range 192–223 → **Class C**

3. Net ID: `221.46.75`

4. Host ID: `64`

Final Answer: Class C, Net ID = `221.46.75`, Host ID = `64`

Ex. 1.3.4 – Subnet Address with /19 Mask**

**Question:** Dest. address = `190.240.34.95`, subnet mask `/19`. Find subnet address.

Steps:

1. `/19` → Decimal mask = `255.255.224.0`

2. In binary:

IP:   10111110.11110000.00100010.01011111

Mask: 11111111.11111111.11100000.00000000

3. Bitwise AND:

Result: 10111110.11110000.00100000.00000000

4. Decimal: `190.240.32.0`

Final Answer: Subnet address = `190.240.32.0`

Ex. 1.3.5 – Subnet Address & Host ID**

Question: Find subnet and host ID for:

a) 120.14.22.16 mask `255.255.128.0`

b) 140.11.36.22 mask `255.255.255.0`

c) 141.181.14.16 mask `255.255.224.0`

d) 200.34.22.156 mask `255.255.255.240`

Steps: Example (a):

1. AND IP with mask:

```

120.14.22.16

255.255.128.0 → 120.14.0.0

```

2. Class A → Net ID = 120.0.0.0, Host ID = 14.0.0

Ex. 1.3.6 – Subnet Mask for 4 Networks

IP: `198.123.46.237` (Class C) → Need 4 subnets.

Steps:

1. Default mask: `255.255.255.0`

2. 4 subnets → need 2 extra bits in mask.

3. New mask = `255.255.255.192`

Final Answer: `255.255.255.192`

Ex. 1.3.7 – Subnet Address

IP: `200.45.34.56`, mask `255.255.240.0`

Steps: Binary AND → `200.45.32.0`

Final Answer: `200.45.32.0`

Ex. 1.3.8 – Designing 6 Subnets (Class C)**

IP: `201.70.64.0`

Steps:

- Default mask = `255.255.255.0`

- Need 3 extra bits → `255.255.255.224`

- 8 subnets possible, use 6.

Ex. 1.3.9 – ≥ 60 Hosts per Subnet**

IP: `195.188.65.0` Class C

Steps:

- Need ≥60 → require 6 host bits (2^6=64-2=62)

- Remaining 2 bits for subnetting → 4 equal subnets.

Ex. 1.3.10 – Max Networks

Class A: 2^7=128

Class B: 2^14=16,384

Class C: 2^21=2,097,152

Ex. 1.3.11 – Max Hosts per Network

A: 2^24=16,777,216

B: 2^16=65,536

C: 2^8=256

Ex. 1.3.12 – Hex to IP

Hex `C2 2F 15 B2` → Decimal `194.79.21.178`

Ex. 1.3.13 – Hosts per Subnet

Class B mask `255.255.240.0` → 12 host bits left → 4096 hosts

Ex. 1.3.14 – Subnetting

Class B mask `255.255.0.0` → Add 3 subnet bits → 6 subnets

Now the 1.4.x, CIDR examples:

Ex. 1.4.1 – Prefix/Suffix for All IPv4

All IPv4 in one block → n=0, suffix=32

Ex. 1.4.2 – Prefix/Suffix for Single Host Blocks

n=32, suffix=0

Ex. 1.4.3 – Network Mask from CIDR

/8 → 255.0.0.0

/16 → 255.255.0.0

Ex. 1.4.4 – Block Info

`64.32.16.8/27`:

n=27 → 32 addresses

First=64.32.16.0

Last=64.32.16.31

Ex. 1.4.5 – Block Info

`129.65.33.1/24`:

n=24 → 256 addresses

First=129.65.33.0

Last=129.65.33.255

Ex. 1.4.6 – Divide Block into 4 Subnets

`130.34.13.64/26` → 64 total, 16 each → new prefix=28

Ex. 1.4.7 – Router CIDR

135.46.63.10 → matches 135.46.60.0/22 → Interface 1

192.53.56.7 → defaults to Router 2