Introduction to IPv6 Addressing

Why IPv6?

IPv4 uses 32-bit addresses → ~4.3 billion unique addresses.

Due to internet growth, IPv4 is not enough.

IPv6 uses 128-bit addresses → provides 3.4 × 10³⁸ addresses (virtually unlimited).

IPv6 Address Structure

An IPv6 address is 128 bits long.

Written as 8 groups of 16-bit blocks, separated by colons (:).

Each block is written in hexadecimal (base 16).

Example:

2001:0db8:0000:0000:0000:ff00:0042:8329

Notations of IPv6 Addressing

1. Colon-Hexadecimal Notation (Standard)

The most common form.

Each 16-bit block is shown in hex, separated by colons.

Example:

2001:0db8:0000:0000:0000:ff00:0042:8329

2. Dotted Decimal Notation

Used only for IPv4-mapped IPv6 addresses.

Embeds an IPv4 address inside IPv6.

Last 32 bits are written in IPv4 dotted-decimal format.

Example:

::ffff:192.168.1.1

Here, 192.168.1.1 is an IPv4 address inside IPv6.

3. Mixed Representation

A combination of hexadecimal and dotted decimal.

Mostly used when IPv4 addresses are embedded in IPv6.

Example:

0:0:0:0:0:ffff:192.168.1.1

4. CIDR Notation (Classless Inter-Domain Routing)

Used to specify prefix length (how many bits represent the network).

Works the same way as IPv4 CIDR.

Example:

2001:db8:0:0:0/32

This means → First 32 bits are network bits, rest for hosts.

Abbreviation Rules in IPv6

Because IPv6 addresses are long, abbreviations help shorten them.

Remove Leading Zeros

2001:0db8:0000:0000:0000:ff00:0042:8329

→ 2001:db8:0:0:0:ff00:42:8329

Use :: to Replace Continuous Zeros

Only once per address!

Example:

2001:db8:0:0:0:ff00:42:8329

→ 2001:db8::ff00:42:8329

Final Shortened Form

2001:0db8:0000:0000:0000:ff00:0042:8329

→ 2001:db8::ff00:42:8329

IP Addressing Concepts

1. Address Space Types

An address space is the total number of unique addresses available.

Unicast

One-to-one communication.

Data is sent from one sender to exactly one receiver.

Example: A computer connecting to a web server.

IPv4 Example: 192.168.1.10

IPv6 Example: 2001:db8::1

Multicast

One-to-many communication.

Data is sent to a group of interested receivers (not everyone).

More efficient than broadcasting.

IPv4 Example: 224.0.0.1 (all systems in the subnet)

IPv6 Example: ff02::1 (all nodes)

Anycast

One-to-nearest communication.

Data is sent to any one member of a group, usually the closest in terms of routing distance.

Common in CDNs (Content Delivery Networks) like Google DNS (8.8.8.8).

IPv6 heavily uses anycast.

2. Address Space Allocation

IP addresses are divided into blocks to manage them effectively.

First Section (IPv4 Classes)

Class A → 1.0.0.0 – 126.0.0.0 (Large networks)

Class B → 128.0.0.0 – 191.255.0.0 (Medium networks)

Class C → 192.0.0.0 – 223.255.255.0 (Small networks)

Class D → 224.0.0.0 – 239.255.255.255 (Multicast)

Class E → 240.0.0.0 – 255.255.255.255 (Experimental)

Second Section (CIDR – Classless Inter-Domain Routing)

Instead of fixed classes, IP addresses are allocated using a prefix length (/n).

Example: 192.168.1.0/24 → 256 addresses.

Example: 10.0.0.0/8 → 16 million addresses.

Algorithm for Allocation

Global (Public) IPs are given by IANA → RIRs (Regional Internet Registries) → ISPs → Users.

Private IPs are reserved for LANs.

IPv4: 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16

IPv6: fc00::/7 (Unique Local Addresses)

3. Assigned / Reserved Blocks

Unspecified Address

Used when the device doesn’t yet know its own address.

IPv4: 0.0.0.0

IPv6: ::

Loopback Address

Used for testing on the same machine.

IPv4: 127.0.0.1

IPv6: ::1

4. Difference Between Loopback in IPv4 vs IPv6

| Feature | IPv4 Loopback | IPv6 Loopback |
| --- | --- | --- |
| Address | 127.0.0.1 | ::1 |
| Block Size | 127.0.0.0/8 (16M addresses) | Just ::1 (single address) |
| Usage | Local testing | Local testing |
| Example | ping 127.0.0.1 | ping ::1 |

5. IPv6 Special Embedded IPv4 Addresses

IPv6 can carry IPv4 addresses for backward compatibility.

Compatible Address (obsolete) → ::192.168.1.1

Mapped Address (for IPv4-to-IPv6 translation) → ::ffff:192.168.1.1

6. Checksum Calculation

Checksums are used to detect errors in transmitted packets.

Algorithm (e.g., IPv4 Header Checksum):

Divide header into 16-bit words.

Add all 16-bit words together.

If overflow occurs, wrap around and add carry.

Take 1’s complement of the result.

Insert it into the checksum field

✅ If receiver computes checksum and gets all 1s (FFFF), then the packet is valid.

Example:

Word1: 0100110010101110

Word2: 1100101001011001

Sum:   1001011100000111

Complement: 0110100011111000 → Checksum