TCP Congestion Control – Slow Start Algorithm

Purpose:

The Slow Start Algorithm is used by TCP to avoid overwhelming the network when a connection begins. It controls how much data the sender can transmit before receiving an acknowledgment (ACK) from the receiver.

How It Works:

Initial Window (cwnd):

When a TCP connection starts, the congestion window (cwnd) is set to 1 Maximum Segment Size (MSS).

This means only 1 segment of data can be sent without acknowledgment initially.

Exponential Growth:

Each time an ACK is received, the cwnd increases exponentially — doubling every Round Trip Time (RTT).

This allows the sender to quickly ramp up its sending rate if the network is free of congestion.

Threshold (ssthresh):

Slow Start continues until the cwnd reaches a threshold called ssthresh (slow start threshold).

After this, TCP switches to Congestion Avoidance, where cwnd grows linearly instead of exponentially.

Step-by-Step Example:

| RTT | cwnd (MSS) | Notes |
| --- | --- | --- |
| 0 | 1 | Initial window |
| 1 | 2 | ACK received for first segment |
| 2 | 4 | ACKs received for 2 segments |
| 3 | 8 | Exponential growth continues |
| … | … | Continue until cwnd = ssthresh |

Key Points:

Fast Growth: Slow Start grows quickly to utilize available network bandwidth.

Avoid Congestion: If packet loss occurs (indicating congestion), cwnd is reduced, and TCP may restart Slow Start or enter Fast Recovery.

Efficiency: Prevents sending too much data at once, reducing the chance of network congestion.

Visualization (Optional for Teaching):

cwnd

8 |          *

7 |

6 |

5 |

4 |       *

3 |

2 |     *

1 |   *

------------------> Time (RTT)

Exponential curve shows cwnd doubling each RTT until ssthresh

TCP Timers

TCP uses several timers to ensure reliable delivery of data, manage congestion, and maintain connections. Here’s a breakdown of the main TCP timers:

1. Retransmission Timer

Purpose: Detect lost packets and retransmit them.

How it works:

Starts a timer when a segment is sent.

If no acknowledgment (ACK) is received before the timer expires, the segment is retransmitted.

Key Point: TCP uses adaptive retransmission, where the timer value is based on estimated Round Trip Time (RTT).

Example:

Segment sent → no ACK after 200 ms → retransmit.

2. Persistence Timer

Purpose: Prevent deadlock when the receiver’s window size is zero.

How it works:

If the sender receives a zero window advertisement (receiver cannot accept more data), the persistence timer ensures the sender periodically checks if the window has opened.

Key Point: Avoids the situation where the sender waits indefinitely.

3. Keepalive Timer

Purpose: Check if an idle connection is still alive.

How it works:

If no data is exchanged for a long period, TCP sends a keepalive probe.

If no response, the connection may be terminated.

Key Point: Helps detect crashed or unreachable peers.

Example:

Idle TCP connection → keepalive probe sent every 2 hours.

4. TIME-WAIT Timer

Purpose: Ensure all packets have left the network before closing a connection.

How it works:

After the connection is closed, TCP waits for 2 × Maximum Segment Lifetime (MSL) before releasing resources.

Key Point: Prevents delayed packets from being misinterpreted as new data.

Example:

Connection closed → wait 2 minutes (if MSL = 1 min) before freeing resources.

Summary Table

| Timer | Purpose | Key Point / Example |
| --- | --- | --- |
| Retransmission Timer | Detect lost packets and retransmit | Retransmit if ACK not received |
| Persistence Timer | Prevent deadlock when receiver window = 0 | Periodically check for available window |
| Keepalive Timer | Check idle connection | Send probe if connection idle |
| TIME-WAIT Timer | Ensure all packets left network before closing | Wait 2 × MSL before freeing resources |